# AXI Interconnect Core User Guide

### UG-CORE-AXI-INTERCON-v1.4

### February 2023 www.elitestek.com

Copyright © 2023. All rights reserved. 易灵思, the 易灵思 logo, the 钛金系列 logo, Quantum, Trion, and Efinity are trademarks of 易灵思, Inc. All other trademarks and service marks are the property of their respective owners. All specifications subject to change without notice.

Contents

Introduction	3

Features	3

Resource Utilization and Performance	3

Functional Description	4

Ports	4

Arbitration Modes	8

Address Decoders	9

AXI Interconnect Operations	10

IP Manager	13

Customizing the AXI Interconnect	14

AXI Interconnect Example Design	15

AXI Interconnect Testbench	16

Revision History	16

# Introduction

The AXI Interconnect core manages traffic on the AXI interfaces where it allows you to connect one or more AXI memory-mapped masters to one or more AXI memory-mapped slaves. For example, when multiple masters are issuing AXI transaction requests simultaneously, the AXI Interconnect core determines which AXI master issues the AXI transaction.

The AXI Interconnect core targets applications such as multiple masters accessing HyperRAM controller using half duplex AXI mode or configurations on multi-slave devices using the same master device.

Use the IP Manager to select IP, customize it, and generate files. The AXI Interconnect core has an interactive wizard to help you set parameters. The wizard also has options to create a testbench and/or example design targeting an 易灵思® development board.

# Features

- Supports AXI3, AXI4, and AXI4-lite interfaces

- Supports N-to-1, 1-to-N, N-to-M (shared access mode) interconnect

- Supports 3 arbitration modes

  - Fixed priority

  - Round robin 1

  - Round robin 2

- Verilog HDL RTL and simulation testbench

#### New in v2022.2

Added pipestage to improve timing critical path.

#### FPGA Support

The AXI Interconnect core supports all Trion® and 钛金系列 FPGAs.

# Resource Utilization and Performance

Note: The resources and performance values provided are just guidance and change depending on the device resource utilization, design congestion, and user design.

#### 钛金系列 Resource Utilization and Performance

| FPGA | Configuration | Logic and Adders | Flip-flops | Memory Blocks | DSP Blocks | fMAX (MHz) | Efinity® Version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ti60 F225 C4 | 1-to-8 with 32-bit data width | 470 | 214 | 0 | 0 | 360 | 2022.1 |
|  | 2-to-1 with 128-bit data width | 910 | 720 | 0 | 0 | 315 |  |

(1) Using Verilog HDL.

# Functional Description

The AXI Interconnect core consists of the following blocks:

- Arbiter—Logic for 3 different arbitration modes

- Skid Buffer—Datapath for write data channel and read data channel

- Address decoders—Decodes address from the incoming AXI transaction and determines the targeted AXI slave transaction

##### Figure 1: AXI Interconnect System Block Diagram

Connected to AXI Master

Write Address Channel (Slave)

Read Address Channel (Master)

Write Data Channel (Master)

Read Data Channel (Master)

Write Response Channel (Master)

Arbiter

Decoder and State Machine

Skid Buffer

AXI Interconnect Core

Read Address Channel (Slave)

Write Data Channel (Slave)

Read Data Channel (Slave)

clk

rstn

Write Response Channel (Slave)

Write Address Channel (Master)

Connected to AXI Slave

## Ports

Note: M represents the number of AXI master ports while S represents the number of AXI slave ports.

Table 1: Global

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| clk | Input |  |  |  | Clock |
| rst_n | Input |  |  |  | Active low asynchronous reset |

Table 2: Write Address Channel (Slave)

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| s_axi_awvalid[S-1:0] | Input |  |  |  | Write address channel valid |
| s_axi_awaddr[S*ADDR_WIDTH-1:0] | Input |  |  |  | Write address channel address |
| s_axi_awprot[S*3-1:0] | Input |  |  |  | Write address channel protect |
| s_axi_awid[S*ID_WIDTH-1:0] | Input |  |  | – | Write address channel transaction ID |
| s_axi_awburst[S*2-1:0] | Input |  |  | – | Write address channel burst type |
| s_axi_awlen[S*8-1:0] | Input |  |  | – | Write address channel burst length |
| s_axi_awsize[S*3-1:0] | Input |  |  | – | Write address channel transfer size |
| s_axi_awcache[S*4-1:0] | Input | – |  | – | Write address channel cache encoding |
| s_axi_awqos[S*4-1:0] | Input | – |  | – | Write address channel Quality of Service (QoS) |
| s_axi_awuser[S*USER_WIDTH-1:0] | Input | – |  | – | Write address channel user- defined signals |
| s_axi_awlock[S*2-1:0] | Input |  | – | – | Write address channel locked transaction |
| s_axi_awready[S-1:0] | Output |  |  |  | Write address channel ready |

Table 3: Write Data Channel (Slave)

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| s_axi_wvalid[S-1:0] | Input |  |  |  | Write channel valid |
| s_axi_wdata[S*DATA_WIDTH-1:0] | Input |  |  |  | Write channel data |
| s_axi_wstrb[S*STRB_WIDTH-1:0] | Input |  |  |  | Write channel strobe (Single bit represents data byte) |
| s_axi_wlast[S-1:0] | Input |  |  | – | Write channel last data beat |
| s_axi_wuser[S*USER_WIDTH-1:0] | Input | – |  | – | Write channel user-defined signals |
| s_axi_wid[S*ID_WIDTH-1:0] | Input |  | – | – | Write channel transaction ID |
| s_axi_wready[S-1:0] | Output |  |  |  | Write channel ready |

Table 4: Write Response Channel (Slave)

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| s_axi_bready[S-1:0] | Input |  |  |  | Write response channel ready |
| s_axi_bresp[S*2-1:0] | Output |  |  |  | Write response channel response |
| s_axi_bvalid[S-1:0] | Output |  |  |  | Write response channel valid |
| s_axi_bid[S*ID_WIDTH-1:0] | Output |  |  | – | Write response channel transaction ID |
| s_axi_buser[S*USER_WIDTH-1:0] | Output | – |  | – | Write response channel user- defined ID |

Table 5: Read Address Channel (Slave)

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| s_axi_arvalid[S-1:0] | Input |  |  |  | Read address channel valid |
| s_axi_araddr[S*ADDR_WIDTH-1:0] | Input |  |  |  | Read address channel address |
| s_axi_arprot[S*3-1:0] | Input |  |  |  | Read address channel protect |
| s_axi_arid[S*ID_WIDTH-1:0] | Input |  |  | – | Read address channel transaction ID |
| s_axi_arburst[S*2-1:0] | Input |  |  | – | Read address channel burst type |
| s_axi_arlen[S*8-1:0] | Input |  |  | – | Read address channel burst length |
| s_axi_arsize[S*3-1:0] | Input |  |  | – | Read address channel transfer size |
| s_axi_arcache[S*4-1:0] | Input | – |  | – | Read address channel cache encoding |
| s_axi_arqos[S*4-1:0] | Input | – |  | – | Read address channel QoS |
| s_axi_aruser[S*USER_WIDTH-1:0] | Input | – |  | – | Read address channel user- defined signals |
| s_axi_arlock[S*2-1:0] | Input |  | – | – | Read address channel locked transaction |
| s_axi_arready[S-1:0] | Output |  |  |  | Read address channel ready |

Table 6: Read Data Channel (Slave)

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| s_axi_rready[S-1:0] | Input |  |  |  | Read data channel ready |
| s_axi_rid[S*ID_WIDTH-1:0] | Output |  |  | – | Read data channel transaction ID |
| s_axi_rdata[S*DATA_WIDTH-1:0] | Output |  |  |  | Read data channel data |
| s_axi_rresp[S*2-1:0] | Output |  |  |  | Read data channel response |
| s_axi_rvalid[S-1:0] | Output |  |  |  | Read data channel valid |
| s_axi_rlast[S-1:0] | Output |  |  | – | Read data channel last data beat |
| s_axi_ruser[S*USER_WIDTH-1:0] | Output | – |  | – | Read data user-defined channel |

Table 7: Write Address Channel (Master)

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| m_axi_awvalid[M-1:0] | Output |  |  |  | Write address channel valid |
| m_axi_awaddr[M*ADDR_WIDTH-1:0] | Output |  |  |  | Write address channel address |
| m_axi_awprot[M*3-1:0] | Output |  |  |  | Write address channel protect |
| m_axi_awid[M*ID_WIDTH-1:0] | Output |  |  | – | Write address channel transaction ID |
| m_axi_awburst[M*2-1:0] | Output |  |  | – | Write address channel burst type |

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| m_axi_awlen[M*8-1:0] | Output |  |  | – | Write address channel burst length |
| m_axi_awsize[M*3-1:0] | Output |  |  | – | Write address channel transfer size |
| m_axi_awcache[M*4-1:0] | Output | – |  | – | Write address channel cache encoding |
| m_axi_awqos[M*4-1:0] | Output | – |  | – | Write address channel QoS |
| m_axi_awuser[M*USER_WIDTH-1:0] | Output | – |  | – | Write address channel user- defined signals |
| m_axi_awlock[M*2-1:0] | Output |  | – | – | Write address channel locked transaction |
| m_axi_awready[M-1:0] | Input |  |  |  | Write address channel ready |

Table 8: Write Data Channel (Master)

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| m_axi_wvalid[M-1:0] | Output |  |  |  | Write channel valid |
| m_axi_wdata[M*DATA_WIDTH-1:0] | Output |  |  |  | Write channel data |
| m_axi_wstrb[M*STRB_WIDTH-1:0] | Output |  |  |  | Write channel strobe (single bit represents data byte) |
| m_axi_wlast[M-1:0] | Output |  |  | – | Write channel last data beat |
| m_axi_wuser[M*USER_WIDTH-1:0] | Output | – |  | – | Write channel user-defined signals |
| m_axi_wid[M*ID_WIDTH-1:0] | Output |  | – | – | Write channel transaction ID |
| m_axi_wready[M-1:0] | Input |  |  |  | Write channel ready |

Table 9: Write Response Channel (Master)

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| m_axi_bready[M-1:0] | Output |  |  |  | Write response channel ready |
| m_axi_bresp[M*2-1:0] | Output |  |  |  | Write response channel response |
| m_axi_bvalid[M-1:0] | Output |  |  |  | Write response channel valid |
| m_axi_bid[M*ID_WIDTH-1:0] | Output |  |  | – | Write response channel transaction ID |
| m_axi_buser[M*USER_WIDTH-1:0] | Input | – |  | – | Write response channel user- defined ID |

Table 10: Read Address Channel (Master)

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| m_axi_arvalid[M-1:0] | Output |  |  |  | Read address channel valid |
| m_axi_araddr[M*ADDR_WIDTH-1:0] | Output |  |  |  | Read address channel address |
| m_axi_arprot[M*3-1:0] | Output |  |  |  | Read address channel protect |
| m_axi_arid[M*ID_WIDTH-1:0] | Output |  |  | – | Read address channel transaction ID |
| m_axi_arburst[M*2-1:0] | Output |  |  | – | Read address channel burst type |
| m_axi_arlen[M*8-1:0] | Output |  |  | – | Read address channel burst length |
| m_axi_arsize[M*3-1:0] | Output |  |  | – | Read address channel transfer size |
| m_axi_arcache[M*4-1:0] | Output | – |  | – | Read address channel cache encoding |
| m_axi_arqos[M*4-1:0] | Output | – |  | – | Read address channel QoS |
| m_axi_aruser[M*USER_WIDTH-1:0] | Output | – |  | – | Read address channel user- defined signals |

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| m_axi_arlock[M*2-1:0] | Output |  | – | – | Read address channel locked transaction |
| m_axi_arready[M-1:0] | Input |  |  |  | Read address channel ready |

Table 11: Read Data Channel (Master)

| Port | Direction | AXI3 | AXI4 | AXI4-Lite | Description |
| --- | --- | --- | --- | --- | --- |
| m_axi_rready[M-1:0] | Output |  |  |  | Read data channel ready |
| m_axi_rid[M*ID_WIDTH-1:0] | Output |  |  | – | Read data channel transaction ID |
| m_axi_rdata[M*DATA_WIDTH-1:0] | Output |  |  |  | Read data channel data |
| m_axi_rresp[M*2-1:0] | Output |  |  |  | Read data channel response |
| m_axi_rvalid[M-1:0] | Output |  |  |  | Read data channel valid |
| m_axi_rlast[M-1:0] | Output |  |  | – | Read data channel last data beat |
| m_axi_ruser[M*USER_WIDTH-1:0] | Input | – |  | – | Read data user-defined channel |

## Arbitration Modes

The AXI Interconnect core includes arbiter engines that grant the request to an AXI master when more than one AXI master issues a request. The AXI Interconnect supports three types of arbitration modes.

#### Fixed Priority

In this mode, the arbiter always prioritizes the most significant bit (MSB) ports as indicated by the bits shown in bold in the following example. Lower priority ports suffer from starvation.

| Request | 11100000 | 11100000 | 11100000 | 1110010 | 00001011 | 00000000 | 00001111 | 11111111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grant | 10000000 | 10000000 | 10000000 | 10000000 | 00001000 | 00000000 | 00001000 | 10000000 |

#### Round Robin 1

The arbitration starts from the MSB port. When more than two transaction requests are issued at the same time, the arbiter grants the request to the port sitting on the right-hand side nearest to the previously served port.

Example:

| Request | 11100000 | 11100000 | 11100000 | 1110010 | 00001011 | 00000000 | 00001111 | 11111111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grant | 10000000 | 01000000 | 00100000 | 00000010 | 00000001 | 00000000 | 00001000 | 00000100 |

#### Round Robin 2

The arbitration starts from the MSB port. The arbitration goes through all connected master ports sequentially with a counter. When there is no request from the assigned master, the arbiter grants the port sitting on the right- hand side nearest to the assigned port. There are no counter increments when there are no requests.

Example:

| Request | 11100000 | 11100000 | 11100000 | 1110010 | 00001011 | 00000000 | 00001111 | 11111111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Counter | 7 | 6 | 5 | 4 | 3 | 3 | 2 | 1 |
| Grant | 10000000 | 01000000 | 00100000 | 00000010 | 00001000 | 00000000 | 00000100 | 00000010 |

## Address Decoders

The address decoder performs address decoding from the incoming AXI transaction address and determines the targeted AXI slave transaction. The AXI Interconnect core does not alter the targeted address content and forwards it entirely to the slave device.

The following table shows an example of AXI slave ports that have a 12-bit address space. The AXI Interconnect core decodes the address and routes the transaction request to the corresponding slave without discarding the address offset.

Table 12: Address Decoder Example

| Original Address | Forwarded Address | Targeted Slave |
| --- | --- | --- |
| 00000FFF | 00000FFF | 0 |
| 00001FFF | 00001FFF | 1 |
| 00002FFF | 00002FFF | 2 |
| 00003FFF | 00003FFF | 3 |

## AXI Interconnect Operations

The following two waveforms illustrate a 2-to-1 interconnect example using fixed priority arbitration mode. There are three requests from two AXI master ports. The AXI Interconnect core grants read operation from port 1 (s_axi_arvalid[1]) because:

- Port 1 has a higher index (MSB)

- A read operation has a higher priority than a write operation

The ready signal of the address channel (s_axi_arready[1]) assertion indicates the request has been accepted. There is one clock cycle latency for the read request to be present on the AXI interconnect output.

##### Figure 2: Fixed Priority Arbitration 2-to-1 Interconnect Example Waveform (Request)

1 clock cycle latency

clk s_axi_arvalid[1] s_axi_awvalid[1] s_axi_arvalid[0] s_axi_awvalid[0] s_axi_arready[1] s_axi_awready[1] s_axi_arready[0] s_axi_awready[0] m_axi_arvalid[0] m_axi_arready[0] m_axi_awvalid[0] m_axi_awready[0]

Read operations, there is one clock cycle latency for the data to be present on the AXI interconnect input.

##### Figure 3: Fixed Priority Arbitration 2-to-1 Interconnect Example Waveform (Read)

read data 1

read data 2

1 clock cycle latency

read data 1

read data 2

clk m_axi_rvalid[0] m_axi_rdata[127:0] s_axi_rvalid[1] s_axi_rdata[255:128]

s_axi_rvalid[0] s_axi_rdata[127:0]

The following waveform illustrates the AXI interconnect of a 1-to-8 operation. The AXI interconnect decodes the slave destination from the write/read address channel and routes the transaction to the corresponding destination. For example, each of the connected slaves has a 12-bit address range.

Address ‘h4000 indicates that the transaction request is targeting slave port 4.

##### Figure 4: 1-to-8 Interconnect Example Waveform

00004000

1 clock cycle latency

Decoded destination

clk s_axi_arvalid[0] s_axi_awvalid[0] s_axi_awaddr[31:0] m_axi_awvalid[7] m_axi_awvalid[6] m_axi_awvalid[5] m_axi_awvalid[4] m_axi_awvalid[3] m_axi_awvalid[2] m_axi_awvalid[1] m_axi_awvalid[0]

m_axi_awready[7:0]	FF

5E4AF16A 02AD58F4 637081C2 4D4CF20A 994..

1 clock cycle latency

5E4AF16A 02AD58F4 637081C2 4D4..

s_axi_wvalid[0] s_axi_wdata[31:0] m_axi_wvalid[7] m_axi_wvalid[6] m_axi_wvalid[5] m_axi_wvalid[4] m_axi_wvalid[3] m_axi_wvalid[2] m_axi_wvalid[1] m_axi_wvalid[0] m_axi_wdata[255:0]

The following waveform illustrates a transaction address request with an invalid range. Each of the connected slaves has a 12-bit address range. There are eight destination slaves configured in the AXI interconnect where the valid decoded address range is 0h0 to 0h7fff. However, the master side

is driving a transaction address to hE000, which is out of the valid address range. The AXI Interconnect core denies the transaction by returning the decoder error.

##### Figure 5: Invalid Transaction Address Example Waveform

clk

M_PORTS   8	 M_ADDR_WIDTH  0000000c_0000000c_0000000c_0000000c_0000000c_0000000c_0000000c_0000000c

0000E000

Decoder error

3

s_axi_arvalid[0:0] s_axi_arready[0:0] s_axi_araddr[31:0] m_axi_arvalid[7] m_axi_arvalid[6] m_axi_arvalid[5] m_axi_arvalid[4] m_axi_arvalid[3] m_axi_arvalid[2] m_axi_arvalid[1] m_axi_arvalid[0] m_axi_rvalid[0:0] s_axi_rresp[1:0] s_axi_rlast[0:0]

# IP Manager

The Efinity® IP Manager is an interactive wizard that helps you customize and generate 易灵思® IP cores. The IP Manager performs validation checks on the parameters you set to ensure that your selections are valid. When you generate the IP core, you can optionally generate an example design targeting an 易灵思 development board and/or a testbench. This wizard is helpful in situations in which you use several IP cores, multiple instances of an IP core with different parameters, or the same IP core for different projects.

Note: Not all 易灵思 IP cores include an example design or a testbench.

#### Generating a Core with the IP Manager

The following steps explain how to customize an IP core with the IP Configuration wizard.

- Open the IP Catalog.

- Choose an IP core and click Next. The IP Configuration wizard opens.

- Enter the module name in the Module Name box.

Note: You cannot generate the core without a module name.

- Customize the IP core using the options shown in the wizard. For detailed information on the options, refer to the IP core's user guide or on-line help.

- (Optional) In the Deliverables tab, specify whether to generate an IP core example design targeting an 易灵思® development board and/or

testbench. For SoCs, you can also optionally generate embedded software example code. These options are turned on by default.

- (Optional) In the Summary tab, review your selections.

- Click Generate to generate the IP core and other selected deliverables.

- In the Review configuration generation dialog box, click Generate. The Console in the Summary tab shows the generation status.

Note: You can disable the Review configuration generation dialog box by turning off the Show Confirmation Box option in the wizard.

- When generation finishes, the wizard displays the Generation Success

dialog box. Click OK to close the wizard.

The wizard adds the IP to your project and displays it under IP in the Project pane.

#### Generated Files

The IP Manager generates these files and directories:

  - <module name>_define.vh—Contains the customized parameters.

  - <module name>_tmpl.v—Verilog HDL instantiation template.

  - <module name>_tmpl.vhd—VHDL instantiation template.

  - <module name>.v—IP source code.

  - settings.json—Configuration file.

  - <kit name>_devkit—Has generated RTL, example design, and Efinity® project targeting a specific development board.

  - Testbench—Contains generated RTL and testbench files.

Note: Refer to the IP Manager chapter of the Efinity® Software User Guide for more information about the Efinity® IP Manager.

# Customizing the AXI Interconnect

The core has parameters so you can customize its function. You set the parameters in the General tab of the core's IP Configuration window.

Table 13: AXI Interconnect Core Parameters (General Tab)

| Parameters | Options | Description |
| --- | --- | --- |
| Protocol | AXI3, AXI4, AXI4-LITE | Defines the AXI Interface Protocol Default: AXI4 |
| Arbitration Mode | PRIORITY, ROUND_ROBIN_1, ROUND_ROBIN_2 | Defines the Arbitration Mode Default: PRIORITY |
| Number of Slave Interfaces | 1 - 8 | Defines the number of slave interface connected to the master port. Default: 2 |
| Number of Master Interfaces | 1 - 8 | Defines the number of master interface connected to the slave port. Default: 1 |
| AXI Address Width | 12 - 64 | Defines the address channel address width. Default: 32 |
| AXI Data Width | 32, 64, 128, 256, 512 | Defines the write/read channel data width. Default: 32 |
| AXI ID Width | 1 - 32 | Defines the ID transaction width. Default: 1 |
| AXI User Width | 1 - 32 | Defines the user define signal width. Default: 1 |

Table 14: AXI Interconnect Core Parameters (AXI Tab)

The number of AXI_S rows depends on the Number of Slave Interfaces parameter you set.

| Parameters | Options | Description |
| --- | --- | --- |
| Destination Address Space (Address Width) | 12 - 32 | Defines the slave address width. You can configure multiple slaves with different address widths by concatenating the parameter. Example: Slave 0 and slave 1 have an address width of 12, slave 2 has an address width of 20, and slave 3 has an address width of 24. The correct parameter setting is {32'd24, 32'd20, 32'd12, 32'd12}. Default: 12 |
| Destination Address Offset (Base Address) | h00001000 - hFFFFF000 | Starting offset address for each slave. Example: The starting offset address for each slave are: - slave 0 (12-bit) : 00000000 |
|  |  | - slave 1 (12-bit) : 00001000 |
|  |  | - slave 2 (20-bit) : 00100000 |
|  |  | - slave 3 (24-bit) : 01000000 |
|  |  | The correct parameter setting is, |
|  |  | {32'h01000000, 32'h00100000, 32'h00001000, 32'h00000000} |

# AXI Interconnect Example Design

You can choose to generate the example design when generating the core in the IP Manager Configuration window. Compile the example design project and download the .hex or .bit file to your board.

Important: 易灵思 tested the example design generated with the default parameter options only.

The example designs target the 钛金系列 Ti60 F225 Development Board. The design implements an AXI Interconnect in the FPGA and demonstrates a single master port access to eight different slave ports. The read and write data is routed by the AXI Interconnect core according to addresses assigned.

##### Figure 6: AXI Interconnect Example Design

Test Done

Test Pass Test Fail

LED D16

AXI-Interconnect Core Example Design

1 AXI Port

8 AXI Ports

Master

AXI Interconnect Core

Slave AXI_S7

Slave AXI_S1

Slave AXI_S0

The master issues eight 128-burst of AXI write data each targeting 8 different slaves. Each slave consists of 128-depth data FIFO making each data transaction to fill up the available FIFO. The master issues read operation to all slaves. Then the master compares read data with the write data from each slave accordingly. The development board LEDs output the following:

Table 15: Example Design Output

| Output |  | Description |
| --- | --- | --- |
| LED D16 Blue | Test Done | Indicates the test is completed. |
| LED D16 Green | Test Pass | Indicates the written and read data are matched. |
| LED D16 Red | Test Fail | Indicates the written and read data are not matched. |

Table 16: Slave Ports Address Range

| Slave | Base Address | Address Width |
| --- | --- | --- |
| AXI_S0 | 0x00000000 | 28 |
| AXI_S1 | 0x10000000 | 24 |
| AXI_S2 | 0x11000000 | 12 |
| AXI_S3 | 0x11100000 | 20 |
| AXI_S4 | 0x20000000 | 28 |
| AXI_S5 | 0x30000000 | 28 |
| AXI_S6 | 0x40000000 | 24 |
| AXI_S7 | 0x41000000 | 20 |

# AXI Interconnect Testbench

You can choose to generate the testbench when generating the core in the IP Manager Configuration window.

Note: You must include all .v files generated in the /testbench directory in your simulation.

易灵思 provides a simulation script for you to run the testbench quickly using the Modelsim software. To run the Modelsim testbench script, run vsim -do modelsim.do in a terminal application. You must have Modelsim installed in your computer to use this script.

The testbench instantiates the IP core and also the simplified AXI master and slave models. The AXI master performs write and read transaction

requests to eight different slave destinations. The AXI Interconnect routes the transaction request to the corresponding slave destination. All the written data is read back through read the data channel and compared throughout the simulation.

# Revision History

Table 17: Revision History

| Date | Version | Description |
| --- | --- | --- |
| February 2023 | 1.4 | Added note about the resource and performance values in the resource and utilization table are for guidance only. |
| December 2022 | 1.3 | Added New in Version topic. |
| August 2022 | 1.2 | Updated for Efinity IP Manager. Added example design section. |
| March 2022 | 1.1 | Update M_ADDR_WIDTH and added M_BASE_ADDR parameters. (DOC-759) |
| February 2022 | 1.0 | Initial release. |